02 - LED Fade

Gradually change LED brightness using PWM. Introduction to analogWrite.

Goal

Fade an LED from off to full brightness in step-wise stages using analogWrite on a PWM-enabled pin. Learn how microcontrollers simulate analog voltages.

What You Will Build

An LED connected to pin D9 steps through distinct brightness levels from 0 to 255 and back down, creating a smooth fading effect.

Why D9? Arduino pins D3, D5, D6, D9, D10, and D11 support Pulse Width Modulation (PWM). D9 is chosen as a standard PWM channel. Pin D13 does not support PWM on the Uno and cannot be used for fading.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| LED | led | Yes | Yes |
| 220 ohm resistor | resistor | Optional | Yes |

Wiring

| LED Pin | Arduino Pin | Notes |
| --- | --- | --- |
| A (Anode, longer leg) | D9 | Signal pin - must be PWM-capable (marked ~) |
| C (Cathode, shorter leg) | GND | Ground reference |

Code

// Pin number for the LED (must be a PWM pin) const int LED_PIN = 9; void setup() { // Configure the pin as an output pinMode(LED_PIN, OUTPUT); // Start serial communication at 9600 baud Serial.begin(9600); Serial.println("LED Fade ready"); } void loop() { // Step up brightness levels (0 is off, 255 is full brightness) analogWrite(LED_PIN, 0); delay(200); analogWrite(LED_PIN, 51); delay(200); analogWrite(LED_PIN, 102); delay(200); analogWrite(LED_PIN, 153); delay(200); analogWrite(LED_PIN, 204); delay(200); analogWrite(LED_PIN, 255); delay(200); // Step down brightness levels back to off analogWrite(LED_PIN, 204); delay(200); analogWrite(LED_PIN, 153); delay(200); analogWrite(LED_PIN, 102); delay(200); analogWrite(LED_PIN, 51); delay(200); }

What to Click in MbedO

Drag Arduino Uno onto the canvas.

Drag LED onto the canvas.

Connect LED A (Anode) to Arduino D9.

Connect LED C (Cathode) to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Expected Output

Serial Monitor:

LED Fade ready

Expected Canvas Behavior

| Time (ms) | analogWrite Value | LED State |
| --- | --- | --- |
| 0 - 200 | 0 | OFF |
| 200 - 400 | 51 | Very Dim |
| 400 - 600 | 102 | Dim |
| 600 - 800 | 153 | Medium |
| 800 - 1000 | 204 | Bright |
| 1000 - 1200 | 255 | Full Brightness |
| 1200 - 2000 | Steps down (204 -> 153 -> 102 -> 51) | Dimming |

The LED on the canvas fades in and out in a repeating loop.

Code Walkthrough

| Line | What It Does |
| --- | --- |
| const int LED_PIN = 9; | Declares a constant for pin D9, which supports PWM. |
| analogWrite(LED_PIN, value) | Writes an analog value (PWM duty cycle) between 0 (always off) and 255 (always on) to the LED. |
| delay(200) | Keeps the LED at the written brightness for 200 milliseconds before moving to the next level. |

Hardware & Safety Concept: Pulse Width Modulation (PWM)

Microcontrollers are digital devices - they can only output 0V (LOW) or 5V (HIGH). To simulate an analog voltage in between, they use Pulse Width Modulation (PWM).

PWM works by switching the output pin ON and OFF incredibly fast (approx. 490 Hz on D9).

If the pin is ON 10% of the time and OFF 90% of the time, the LED appears dim.

If the pin is ON 90% of the time and OFF 10% of the time, the LED appears bright.

The percentage of time the signal is HIGH is called the duty cycle. An analogWrite value of 128 corresponds to a 50% duty cycle.

Try This! (Challenges)

Smoother Fading: Increase the number of steps. Add values like 25, 76, 128, 179, and 230 to make the fade feel smoother.

Speed Dial: Adjust all delay(200) values to delay(50) to speed up the fade, or delay(500) to slow it down.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| LED blinks fully on and off instead of fading | Pin doesn't support PWM | Ensure the LED is connected to D9, not D13 or other non-PWM pin. |
| LED does not turn on | Reverse polarity | Connect Anode (A) to D9 and Cathode (C) to GND. |
| LED stays dim or doesn't fade | Missing delay steps | Make sure you have delay(200) after every single analogWrite statement. |

Mode Notes

These patterns (pinMode, digitalWrite, delay, Serial.begin, Serial.println, analogWrite) are supported by MbedO interpreted mode and are safe for this beginner project. This step-wise implementation avoids using for loops, which are not supported by the interpreter.

Related Projects

[01 - LED Blink](01-led-blink.md)

[03 - Breathing LED](03-breathing-led.md)

[14 - Dial LED Brightness](14-dial-led-brightness.md)