03 - Breathing LED

Make an LED smoothly pulse up and down, mimicking a breathing rhythm.

Goal

Learn how to create smooth transitions in LED brightness using a high-density sequence of PWM outputs.

What You Will Build

An LED connected to pin D9 smoothly brightens and dims in a cyclic, non-blocking rhythm.

Why D9? Fading effects require PWM pins. D9 is used to handle the quick, step-wise power updates cleanly.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| LED | led | Yes | Yes |
| 220 ohm resistor | resistor | Optional | Yes |

Wiring

| LED Pin | Arduino Pin | Notes |
| --- | --- | --- |
| A (Anode, longer leg) | D9 | Signal pin - must be PWM-capable |
| C (Cathode, shorter leg) | GND | Ground reference |

Code

const int LED_PIN = 9; void setup() { pinMode(LED_PIN, OUTPUT); Serial.begin(9600); Serial.println("Breathing LED ready"); } void loop() { // Fade In analogWrite(LED_PIN, 0); delay(80); analogWrite(LED_PIN, 25); delay(80); analogWrite(LED_PIN, 51); delay(80); analogWrite(LED_PIN, 76); delay(80); analogWrite(LED_PIN, 102); delay(80); analogWrite(LED_PIN, 128); delay(80); analogWrite(LED_PIN, 153); delay(80); analogWrite(LED_PIN, 179); delay(80); analogWrite(LED_PIN, 204); delay(80); analogWrite(LED_PIN, 230); delay(80); analogWrite(LED_PIN, 255); delay(80); // Fade Out analogWrite(LED_PIN, 230); delay(80); analogWrite(LED_PIN, 204); delay(80); analogWrite(LED_PIN, 179); delay(80); analogWrite(LED_PIN, 153); delay(80); analogWrite(LED_PIN, 128); delay(80); analogWrite(LED_PIN, 102); delay(80); analogWrite(LED_PIN, 76); delay(80); analogWrite(LED_PIN, 51); delay(80); analogWrite(LED_PIN, 25); delay(80); }

What to Click in MbedO

Drag Arduino Uno onto the canvas.

Drag LED onto the canvas.

Connect LED A (Anode) to Arduino D9.

Connect LED C (Cathode) to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Expected Output

Serial Monitor:

Breathing LED ready

Expected Canvas Behavior

| Time (ms) | Direction | Brightness State |
| --- | --- | --- |
| 0 - 880 | Rising | Dim to full brightness |
| 880 - 1600 | Falling | Full brightness back to dim |
| 1600+ | Cycles | Repeats continuously |

Code Walkthrough

| Line | What It Does |
| --- | --- |
| analogWrite(LED_PIN, value) | Sends PWM duty cycle steps of approximately 10% increments (0, 25, 51, etc.) up to 100% (255) and back. |
| delay(80) | Holds each brightness level for 80 ms, creating a transition period of about 1.6 seconds per breath. |

Hardware & Safety Concept: Step Density

In human perception, brightness scaling is non-linear (logarithmic). A sequence of small, equal steps (e.g. increments of 25) with quick delays feels like a smooth, continuous fade. If the step sizes are too large (e.g. jumping from 0 to 128 to 255), the transition will look jerky.

Try This! (Challenges)

Adjust Breath Rate: Change the delay(80) to delay(120) to simulate a slower, relaxed breath, or delay(40) for a fast, panting effect.

Asymmetric Breathing: Change the fade-in delays to delay(60) and the fade-out delays to delay(120) to create a quick inhalation and a slow exhalation.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| LED flashes abruptly | LED pin is wrong | Move the LED anode wire to D9 (PWM). |
| LED stays completely on or off | Missing loop cycle delays | Check that a delay(80) is present after every single analogWrite. |

Mode Notes

These patterns (pinMode, digitalWrite, delay, Serial.begin, Serial.println, analogWrite) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[02 - LED Fade](02-led-fade.md)

[04 - RGB Color Mixer](04-rgb-color-mixer.md)

[14 - Dial LED Brightness](14-dial-led-brightness.md)