04 - RGB Color Mixer

Cycle through red, green, blue, and mixed colors using a common-cathode RGB LED.

Goal

Control three individual color channels (Red, Green, and Blue) of a single RGB LED to produce solid colors and secondary mixed colors.

What You Will Build

A common-cathode RGB LED connected to PWM pins D9, D10, and D11 cycles through six colors - red, green, blue, yellow, cyan, and magenta - pausing for one second at each color.

Why D9, D10, and D11? These three adjacent pins on the Uno all support PWM, which allows for analog brightness control of each color channel individually.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| RGB LED | rgbled | Yes | Yes |
| 220 ohm resistor | resistor | Optional | Yes (3 required) |

Wiring

| RGB LED Pin | Arduino Pin | Notes |
| --- | --- | --- |
| R (Red Anode) | D9 | Red channel control |
| G (Green Anode) | D10 | Green channel control |
| B (Blue Anode) | D11 | Blue channel control |
| C (Common Cathode) | GND | Common ground connection |

> On real hardware, always place a 220 ohm resistor in series on each of the R, G, and B signal lines to prevent burnout.

Code

const int R_PIN = 9; const int G_PIN = 10; const int B_PIN = 11; void setup() { pinMode(R_PIN, OUTPUT); pinMode(G_PIN, OUTPUT); pinMode(B_PIN, OUTPUT); Serial.begin(9600); Serial.println("RGB Color Mixer ready"); } void loop() { // Red analogWrite(R_PIN, 255); analogWrite(G_PIN, 0); analogWrite(B_PIN, 0); Serial.println("Red"); delay(1000); // Green analogWrite(R_PIN, 0); analogWrite(G_PIN, 255); analogWrite(B_PIN, 0); Serial.println("Green"); delay(1000); // Blue analogWrite(R_PIN, 0); analogWrite(G_PIN, 0); analogWrite(B_PIN, 255); Serial.println("Blue"); delay(1000); // Yellow (Red + Green) analogWrite(R_PIN, 255); analogWrite(G_PIN, 255); analogWrite(B_PIN, 0); Serial.println("Yellow"); delay(1000); // Cyan (Green + Blue) analogWrite(R_PIN, 0); analogWrite(G_PIN, 255); analogWrite(B_PIN, 255); Serial.println("Cyan"); delay(1000); // Magenta (Red + Blue) analogWrite(R_PIN, 255); analogWrite(G_PIN, 0); analogWrite(B_PIN, 255); Serial.println("Magenta"); delay(1000); }

What to Click in MbedO

Drag Arduino Uno onto the canvas.

Drag RGB LED onto the canvas.

Connect RGB LED R to Arduino D9.

Connect RGB LED G to Arduino D10.

Connect RGB LED B to Arduino D11.

Connect RGB LED C to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Expected Output

Serial Monitor:

RGB Color Mixer ready Red Green Blue Yellow Cyan Magenta ...

Expected Canvas Behavior

| Color Mode | Red Pin (D9) | Green Pin (D10) | Blue Pin (D11) | Result |
| --- | --- | --- | --- | --- |
| Red | HIGH (255) | LOW (0) | LOW (0) | Red |
| Green | LOW (0) | HIGH (255) | LOW (0) | Green |
| Blue | LOW (0) | LOW (0) | HIGH (255) | Blue |
| Yellow | HIGH (255) | HIGH (255) | LOW (0) | Yellow |
| Cyan | LOW (0) | HIGH (255) | HIGH (255) | Cyan |
| Magenta | HIGH (255) | LOW (0) | HIGH (255) | Magenta |

The RGB LED on the canvas changes color sequentially every 1 second.

Code Walkthrough

| Line | What It Does |
| --- | --- |
| pinMode (setup) | Sets all three control pins (D9, D10, D11) as outputs. |
| analogWrite(R_PIN, 255) | Sets the red channel to 100% duty cycle. |
| analogWrite(G_PIN, 255) | Sets the green channel to 100% duty cycle, combining with red to create yellow light. |

Hardware & Safety Concept: Additive Color Mixing

The RGB LED is actually three tiny, independent LEDs (Red, Green, and Blue) packaged into a single plastic housing.

A Common Cathode RGB LED connects all three negative terminals to a single pin (GND), requiring positive voltage on the R, G, and B pins to light up.

Additive Color Mixing blends these three primary light colors. By adjusting the PWM duty cycle of each pin, you can mix thousands of custom colors (e.g. Red + Green = Yellow).

Try This! (Challenges)

White Light Mixer: Mix all three colors to create white light. (Hint: write 255 to all three pins).

Orange & Purple: Try custom mixtures like Orange (R=255, G=128, B=0) or Purple (R=128, G=0, B=128).

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Colors are swapped | R, G, or B wires are crossed | Check the wiring table: R to D9, G to D10, B to D11. |
| LED is completely off | Common cathode (C) pin is not grounded | Ensure the center/longest leg (C) is connected to a GND pin on the Arduino. |

Mode Notes

These patterns (analogWrite calls sequentially in a flat structure) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[03 - Breathing LED](03-breathing-led.md)

[06 - WS2812B Color Cycle](06-ws2812b-color-cycle.md)