05 - Traffic Light Sequence

Simulate a road traffic light that cycles through red, yellow, and green.

Goal

Control three discrete LEDs (Red, Yellow, and Green) in a timed sequence to model a standard automotive traffic light.

What You Will Build

A pre-packaged Traffic Light module connected to pins D9, D10, and D11 cycles through standard phases: Red (Stop) for 4 seconds -> Green (Go) for 4 seconds -> Yellow (Prepare to Stop) for 1.5 seconds -> repeating.

Why D9, D10, and D11? These pins provide clean digital output controls. They are kept close together to keep canvas wiring organized.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| Traffic Light | traffic_light | Yes | Yes |
| 220 ohm resistor | resistor | Optional | Yes (3 required if wiring discrete LEDs) |

Wiring

| Traffic Light Pin | Arduino Pin | Notes |
| --- | --- | --- |
| RED | D9 | Red LED control line |
| YELLOW | D10 | Yellow LED control line |
| GREEN | D11 | Green LED control line |
| GND | GND | Ground connection |

Code

const int RED_PIN = 9; const int YELLOW_PIN = 10; const int GREEN_PIN = 11; void setup() { pinMode(RED_PIN, OUTPUT); pinMode(YELLOW_PIN, OUTPUT); pinMode(GREEN_PIN, OUTPUT); Serial.begin(9600); Serial.println("Traffic Light ready"); } void loop() { // Red - stop (4 seconds) digitalWrite(RED_PIN, HIGH); digitalWrite(YELLOW_PIN, LOW); digitalWrite(GREEN_PIN, LOW); Serial.println("RED - Stop"); delay(4000); // Green - go (4 seconds) digitalWrite(RED_PIN, LOW); digitalWrite(YELLOW_PIN, LOW); digitalWrite(GREEN_PIN, HIGH); Serial.println("GREEN - Go"); delay(4000); // Yellow - slow down / prepare to stop (1.5 seconds) digitalWrite(RED_PIN, LOW); digitalWrite(YELLOW_PIN, HIGH); digitalWrite(GREEN_PIN, LOW); Serial.println("YELLOW - Slow"); delay(1500); }

What to Click in MbedO

Drag Arduino Uno onto the canvas.

Drag Traffic Light onto the canvas.

Connect Traffic Light RED to Arduino D9.

Connect Traffic Light YELLOW to Arduino D10.

Connect Traffic Light GREEN to Arduino D11.

Connect Traffic Light GND to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Expected Output

Serial Monitor:

Traffic Light ready RED - Stop GREEN - Go YELLOW - Slow RED - Stop ...

Expected Canvas Behavior

| Sequence Phase | Red Pin (D9) | Yellow Pin (D10) | Green Pin (D11) | Active Light | Duration |
| --- | --- | --- | --- | --- | --- |
| 1. Stop | HIGH | LOW | LOW | RED | 4.0 seconds |
| 2. Go | LOW | LOW | HIGH | GREEN | 4.0 seconds |
| 3. Warn | LOW | HIGH | LOW | YELLOW | 1.5 seconds |

The traffic light component on the canvas cycles colors in order.

Code Walkthrough

| Line | What It Does |
| --- | --- |
| pinMode (setup) | Sets all three traffic light pins as outputs. |
| digitalWrite(RED_PIN, HIGH) | Activates the Red LED. |
| digitalWrite(YELLOW_PIN, LOW) | Ensures the Yellow LED is off during the red phase. |
| delay(4000) | Keeps the current state active for 4 seconds. |

Hardware & Safety Concept: Discrete vs Integrated Components

A Traffic Light Module is a convenient integrated board. It mounts three discrete LEDs, three surface-mount resistors (often 330 ohm), and routes their grounds to a single shared pin. If you were building this using individual LEDs, you would have to wire three separate ground paths and three individual resistors manually.

Try This! (Challenges)

UK Signal Sequence: In many countries, the sequence transitions from Red -> Red + Yellow together -> Green -> Yellow -> Red. Modify the code to implement this Red+Yellow transition stage for 1 second.

Pedestrian Crossing Beep: Add a Buzzer to D8 and generate a slow repeating beep (tone(8, 800, 100); delay(1000);) during the green phase to simulate a pedestrian crosswalk signal.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| LEDs light up in the wrong order | Signal control wires crossed | Check the wiring table: RED to D9, YELLOW to D10, GREEN to D11. |
| All three lights stay ON together | Code logic error | Make sure each phase explicitly sets the other two unused pins to LOW when turning one HIGH. |

Mode Notes

These patterns (digitalWrite sequences with delay) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[01 - LED Blink](01-led-blink.md)

[04 - RGB Color Mixer](04-rgb-color-mixer.md)