07 - Buzzer Beep

Make a buzzer produce a single beep tone using tone() and noTone().

Goal

Learn how to use the built-in Arduino tone() and noTone() functions to generate sound frequencies on a digital output pin.

What You Will Build

A piezo buzzer connected to pin D8 plays a 1000 Hz beep for 500 ms, pauses for 500 ms, and repeats the cycle indefinitely.

Why D8? Buzzers can run on any digital I/O pin. D8 is used to keep the wiring simple and distinct from LED output pins.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| Buzzer | buzzer | Yes | Yes |
| 100 ohm resistor | resistor | Optional | Yes (optional, protects buzzer) |

Wiring

| Buzzer Pin | Arduino Pin | Notes |
| --- | --- | --- |
| Pin 1 (Positive / Signal) | D8 | Audio control line |
| Pin 2 (Negative / Ground) | GND | Ground connection |

Code

const int BUZZ_PIN = 8; void setup() { Serial.begin(9600); Serial.println("Buzzer Beep ready"); } void loop() { // Generate a 1000 Hz (1 kHz) frequency tone tone(BUZZ_PIN, 1000); Serial.println("BEEP"); delay(500); // Sound duration // Stop generating the tone noTone(BUZZ_PIN); delay(500); // Silent duration }

What to Click in MbedO

Drag Arduino Uno onto the canvas.

Drag Buzzer onto the canvas.

Connect Buzzer pin 1 to Arduino D8.

Connect Buzzer pin 2 to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Expected Output

Serial Monitor:

Buzzer Beep ready BEEP BEEP ...

Expected Canvas Behavior

| Time (ms) | Output State | Tone Output |
| --- | --- | --- |
| 0 - 500 | tone(8, 1000) | 1000 Hz Tone (Active) |
| 500 - 1000 | noTone(8) | Silent (Inactive) |
| 1000+ | Cycles | Alternates ON/OFF |

The buzzer component on the canvas flashes/vibrates when active.

Code Walkthrough

| Line | What It Does |
| --- | --- |
| tone(BUZZ_PIN, 1000) | Starts a square wave of 1000 Hz on D8 to vibrate the internal buzzer element. |
| noTone(BUZZ_PIN) | Stops the sound wave generation. Without this, the buzzer will make sound forever. |

Hardware & Safety Concept: Piezo Buzzers

A Piezoelectric Buzzer contains a small ceramic disk bonded to a metal plate. When you apply an oscillating voltage, the ceramic bends, creating sound pressure waves.

Active Buzzers have internal oscillators and buzz whenever DC power is connected.

Passive Buzzers (like the one used in MbedO) require an external AC/PWM wave to oscillate. Using the tone() function allows us to specify the exact pitch (frequency) of the sound.

Try This! (Challenges)

Change the Pitch: Modify the frequency parameter in tone() from 1000 to 440 (lower pitch) and then to 3000 (higher chirp).

Double-Beep Pattern: Modify the loop code to play two short 100 ms beeps followed by a 1-second silence.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| No sound is produced | Ground connection missing | Verify Buzzer Pin 2 is connected to a GND pin on the Arduino. |
| Continuous sound with no breaks | Missing noTone call | Make sure you call noTone(BUZZ_PIN) after the first delay, and check for a second delay after noTone. |
| Click sound instead of tone | Signal pin on Ground | Swap the buzzer wires: Pin 1 to D8, Pin 2 to GND. |

Mode Notes

These patterns (tone and noTone calls) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[08 - Buzzer Melody](08-buzzer-melody.md)

[09 - Doorbell Tone](09-doorbell-tone.md)

[10 - Alarm Oscillate](10-alarm-oscillate.md)