08 - Buzzer Melody

Play a short musical melody on a buzzer using a sequence of tone() calls.

Goal

Learn how to program a musical scale by sequencing frequency values and note durations using simple, non-array structures.

What You Will Build

A buzzer connected to pin D8 plays a standard C major scale (8 notes: C4 to C5) once per loop iteration, with a 1-second pause between repetitions.

Why D8? D8 acts as our audio signal line. The buzzer is powered entirely by the digital pulses generated on D8.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| Buzzer | buzzer | Yes | Yes |
| 100 ohm resistor | resistor | Optional | Yes |

Wiring

| Buzzer Pin | Arduino Pin | Notes |
| --- | --- | --- |
| Pin 1 (Positive / Signal) | D8 | Audio control line |
| Pin 2 (Negative / Ground) | GND | Ground connection |

Code

const int BUZZ_PIN = 8; void setup() { Serial.begin(9600); Serial.println("Buzzer Melody ready"); } void loop() { // C major scale notes: C4, D4, E4, F4, G4, A4, B4, C5 // Note 1: C4 (262 Hz) tone(BUZZ_PIN, 262); delay(300); noTone(BUZZ_PIN); delay(50); // Note 2: D4 (294 Hz) tone(BUZZ_PIN, 294); delay(300); noTone(BUZZ_PIN); delay(50); // Note 3: E4 (330 Hz) tone(BUZZ_PIN, 330); delay(300); noTone(BUZZ_PIN); delay(50); // Note 4: F4 (349 Hz) tone(BUZZ_PIN, 349); delay(300); noTone(BUZZ_PIN); delay(50); // Note 5: G4 (392 Hz) tone(BUZZ_PIN, 392); delay(300); noTone(BUZZ_PIN); delay(50); // Note 6: A4 (440 Hz) tone(BUZZ_PIN, 440); delay(300); noTone(BUZZ_PIN); delay(50); // Note 7: B4 (494 Hz) tone(BUZZ_PIN, 494); delay(300); noTone(BUZZ_PIN); delay(50); // Note 8: C5 (523 Hz) tone(BUZZ_PIN, 523); delay(300); noTone(BUZZ_PIN); delay(50); Serial.println("Melody played"); delay(1000); // Pause before repeating }

What to Click in MbedO

Drag Arduino Uno onto the canvas.

Drag Buzzer onto the canvas.

Connect Buzzer pin 1 to Arduino D8.

Connect Buzzer pin 2 to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Expected Output

Serial Monitor:

Buzzer Melody ready Melody played Melody played ...

Expected Canvas Behavior

| Step / Note | frequency (Hz) | Duration (ms) | Inter-Note Silence |
| --- | --- | --- | --- |
| 1. C4 | 262 | 300 | 50 ms |
| 2. D4 | 294 | 300 | 50 ms |
| 3. E4 | 330 | 300 | 50 ms |
| 4. F4 | 349 | 300 | 50 ms |
| 5. G4 | 392 | 300 | 50 ms |
| 6. A4 | 440 | 300 | 50 ms |
| 7. B4 | 494 | 300 | 50 ms |
| 8. C5 | 523 | 300 | 50 ms |

The canvas buzzer visualizes audio output in quick bursts.

Code Walkthrough

| Line | What It Does |
| --- | --- |
| tone(BUZZ_PIN, 262) | Plays a Middle C note (262 Hz) on pin D8. |
| delay(300) | Keeps the note playing for 300 ms. |
| noTone(BUZZ_PIN) | Stops the note. |
| delay(50) | Pauses for 50 ms. This silence gap is crucial; without it, consecutive notes blend together into a single sound. |

Hardware & Safety Concept: Note Pitch and Audio Frequencies

Pitch is determined by the frequency of the sound wave (measured in Hertz, or cycles per second).

Lower numbers (e.g. 262 Hz) have longer wavelengths and sound lower to human ears.

Higher numbers (e.g. 523 Hz) vibrate faster and sound higher.

The tone() function generates a square wave of the exact frequency specified, causing the buzzer's ceramic disk to vibrate at that frequency.

Try This! (Challenges)

Change the Tempo: Change all delay(300) values to delay(150) and see how the speed of the scale changes.

Arpeggio Pattern: Create a custom melody by rearranging the notes. Play C4 (262), E4 (330), G4 (392), and C5 (523) in a jumping sequence.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| The notes sound muddy or run together | Missing inter-note pauses | Ensure the noTone(BUZZ_PIN); delay(50); line is present between every single tone() call. |
| The scale plays too slowly | Tone delay is set too high | Lower the delay after the tone() call (e.g. from 300 ms to 200 ms). |

Mode Notes

Sequenced tone() and noTone() calls with fixed parameters are supported by MbedO interpreted mode. Writing a loop that uses dynamic array indexing would trigger an interpreter warning, which is why a flat, step-wise list of notes is used.

Related Projects

[07 - Buzzer Beep](07-buzzer-beep.md)

[09 - Doorbell Tone](09-doorbell-tone.md)

[10 - Alarm Oscillate](10-alarm-oscillate.md)