09 - Doorbell Tone

Play a two-tone "ding-dong" doorbell sound when a button is pressed.

Goal

Learn how to use a digital input (push button) to trigger a timed sequence of acoustic tones on a buzzer.

What You Will Build

Pressing the push button connected to pin D2 triggers a classic two-note doorbell sequence (E5 at 659 Hz, then C5 at 523 Hz) on a buzzer connected to pin D8.

Why D2 and D8? D2 supports hardware interrupts (useful for buttons in advanced sketches), and D8 handles the audio output signal cleanly.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| Push Button | button | Yes | Yes |
| Buzzer | buzzer | Yes | Yes |
| 100 ohm resistor | resistor | Optional | Yes (optional, protects buzzer) |

Wiring

| Component | Component Pin | Arduino Pin | Notes |
| --- | --- | --- | --- |
| Push Button | 1.r | D2 | Input signal pin |
| Push Button | 2.r | GND | Ground connection |
| Buzzer | Pin 1 (Positive) | D8 | Audio control line |
| Buzzer | Pin 2 (Negative) | GND | Ground connection |

Code

const int BTN_PIN = 2; const int BUZZ_PIN = 8; void setup() { // Use INPUT_PULLUP to keep the input HIGH when button is unpressed pinMode(BTN_PIN, INPUT_PULLUP); Serial.begin(9600); Serial.println("Doorbell ready"); } void loop() { // Read the button state. Under INPUT_PULLUP, LOW means pressed. if (digitalRead(BTN_PIN) == LOW) { Serial.println("Ding dong!"); // Play "Ding" (Note E5: 659 Hz) tone(BUZZ_PIN, 659); delay(400); noTone(BUZZ_PIN); delay(100); // Brief pause // Play "Dong" (Note C5: 523 Hz) tone(BUZZ_PIN, 523); delay(600); noTone(BUZZ_PIN); // Delay to prevent re-triggering immediately delay(500); } }

What to Click in MbedO

Drag Arduino Uno, Push Button, and Buzzer onto the canvas.

Connect Button 1.r to Arduino D2 and 2.r to Arduino GND.

Connect Buzzer pin 1 to Arduino D8 and pin 2 to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Click the button on the canvas once to trigger the doorbell sound.

Expected Output

Serial Monitor when button is pressed:

Doorbell ready Ding dong!

Expected Canvas Behavior

| Button Action | Pin D2 Reading | Buzzer Sequence | Active Pitch |
| --- | --- | --- | --- |
| Not pressed | HIGH | Silent | None |
| Pressed (Click) | LOW | Starts doorbell | E5 (659 Hz) -> C5 (523 Hz) |
| Held down | LOW | Loops sequence | Cycles with 500 ms gap |

The canvas buzzer visualizes the sound waves when the button is active.

Code Walkthrough

| Line | What It Does |
| --- | --- |
| pinMode(BTN_PIN, INPUT_PULLUP) | Configures pin D2 as an input with the internal pull-up resistor active, avoiding a floating pin state. |
| if (digitalRead(BTN_PIN) == LOW) | Triggers the sound sequence only when the pin is pulled to Ground (button pressed). |
| tone(BUZZ_PIN, 659) | Plays the first note (Ding) at 659 Hz. |
| noTone(BUZZ_PIN) | Stops the wave generator between notes. |

Hardware & Safety Concept: Pull-Up Logic

When a pin is configured with INPUT_PULLUP, the microcontroller connects it to 5V through an internal resistor. This defaults the pin to HIGH. When the button is pressed, it forms a direct path to Ground (GND), pulling the voltage down to 0V (LOW). This is called negative logic (LOW = active).

Try This! (Challenges)

Three-Tone Doorbell: Add a third note to create a "ding-dong-ding" sequence. (Hint: Add note G5 at 784 Hz at the end).

Double Doorbell Lockout: Increase the delay at the end of the loop to delay(3000) to prevent people from ringing the doorbell repeatedly.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Doorbell plays constantly without stopping | Button pin is floating | Ensure INPUT_PULLUP is declared in setup(). Check that Button Pin 2.r connects to GND. |
| No sound plays when button is clicked | Ground connection missing | Verify Button 2.r is connected to GND and Buzzer Pin 2 is connected to GND. |
| Sound pitch is wrong | Frequencies incorrect | Verify the frequency arguments match E5 (659) and C5 (523). |

Mode Notes

These patterns (digitalRead conditional checking triggering sequential tone calls) are supported by MbedO interpreted mode.

Related Projects

[07 - Buzzer Beep](07-buzzer-beep.md)

[08 - Buzzer Melody](08-buzzer-melody.md)

[11 - Button ON/OFF](11-button-onoff.md)