10 - Alarm Oscillate

Produce a wailing siren sound by alternating between two frequencies rapidly.

Goal

Learn how to create a continuous siren effect by switching the output frequency of a buzzer repeatedly.

What You Will Build

A buzzer connected to pin D8 alternates between a low tone (500 Hz) and a high tone (1500 Hz) every 300 ms to simulate an emergency alarm siren.

Why D8? Keeping the sound generation on D8 aligns with all beginner audio projects.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| Buzzer | buzzer | Yes | Yes |
| 100 ohm resistor | resistor | Optional | Yes |

Wiring

| Buzzer Pin | Arduino Pin | Notes |
| --- | --- | --- |
| Pin 1 (Positive / Signal) | D8 | Audio control line |
| Pin 2 (Negative / Ground) | GND | Ground connection |

Code

const int BUZZ_PIN = 8; void setup() { Serial.begin(9600); Serial.println("Alarm Oscillate ready"); } void loop() { // Play low-pitch wail (500 Hz) tone(BUZZ_PIN, 500); delay(300); // Play high-pitch wail (1500 Hz) tone(BUZZ_PIN, 1500); delay(300); }

What to Click in MbedO

Drag Arduino Uno onto the canvas.

Drag Buzzer onto the canvas.

Connect Buzzer pin 1 to Arduino D8.

Connect Buzzer pin 2 to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Expected Output

Serial Monitor:

Alarm Oscillate ready

Expected Canvas Behavior

| Loop Step | Tone Pitch | Duration | Sound Character |
| --- | --- | --- | --- |
| 1 | 500 Hz | 300 ms | Low pitch |
| 2 | 1500 Hz | 300 ms | High pitch |
| Repeats | Cycles | 600 ms total | Wailing siren |

The buzzer vibrates and flashes on the canvas continuously.

Code Walkthrough

| Line | What It Does |
| --- | --- |
| tone(BUZZ_PIN, 500) | Generates a 500 Hz tone on D8. |
| delay(300) | Maintains the 500 Hz tone for 300 ms. |
| tone(BUZZ_PIN, 1500) | Immediately switches the output frequency on D8 to 1500 Hz. (No noTone is needed here because the new frequency overrides the old one). |

Hardware & Safety Concept: Frequency Sweeps

In professional sirens, the tone doesn't jump instantly between two pitches (which is a dual-tone alarm). Instead, it sweeps continuously up and down (a wobble/siren alarm). A sweep requires using variables in loops to increment the pitch (e.g. 500 -> 501 -> 502 -> ...). Since loops are not supported in interpreted mode, alternating between two distinct frequencies is the best way to simulate a high-visibility alert sound.

Try This! (Challenges)

Faster Siren: Reduce the delays to 100 ms. Notice how the alarm sounds more urgent.

Three-Tone Siren: Add a third tone of 1000 Hz in the middle to create a cascading alarm sound.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| The pitch does not change | Second tone call is missing | Ensure both tone(BUZZ_PIN, 500) and tone(BUZZ_PIN, 1500) are present with delays after each. |
| Sound is very quiet or rasping | PWM interference | Ensure pin 8 is used. If sharing with other timers, some interference can occur. |

Mode Notes

Alternating two static tone() calls with delays is supported by MbedO interpreted mode.

Related Projects

[07 - Buzzer Beep](07-buzzer-beep.md)

[08 - Buzzer Melody](08-buzzer-melody.md)

[31 - Motion Alarm](../intermediate/31-motion-alarm.md)