11 - Button ON/OFF

Turn an LED on and off by pressing a push button.

Goal

Learn how to read a digital input (a button) and use it to control a digital output (an LED) with conditional statements.

What You Will Build

Pressing and holding the button turns the LED on. Releasing the button turns the LED off.

Why D2 and D13? Pin D2 supports hardware interrupts on the Arduino Uno, which makes it a preferred choice for push button inputs. Pin D13 is used because it maps directly to the Uno's onboard LED, allowing quick verification.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| Push Button | button | Yes | Yes |
| LED | led | Yes | Yes |
| 220 ohm resistor | resistor | Optional | Yes |

Wiring

| Component | Component Pin | Arduino Pin | Notes |
| --- | --- | --- | --- |
| Push Button | 1.r | D2 | Input signal connection |
| Push Button | 2.r | GND | Ground reference |
| LED | A (Anode, longer leg) | D13 | Output signal connection |
| LED | C (Cathode, shorter leg) | GND | Ground reference |

Code

const int BUTTON_PIN = 2; const int LED_PIN = 13; void setup() { // Use INPUT_PULLUP to use the Arduino's internal pull-up resistor pinMode(BUTTON_PIN, INPUT_PULLUP); pinMode(LED_PIN, OUTPUT); Serial.begin(9600); Serial.println("Button ON/OFF Ready"); } void loop() { // Read the state of the button int buttonState = digitalRead(BUTTON_PIN); // Under INPUT_PULLUP, LOW means the button is pressed (connected to GND) if (buttonState == LOW) { digitalWrite(LED_PIN, HIGH); Serial.println("Button Pressed - LED ON"); } else { digitalWrite(LED_PIN, LOW); } delay(50); // Small delay to debounce the signal }

What to Click in MbedO

Drag Arduino Uno, Push Button, and LED onto the canvas.

Connect LED A (Anode) to Arduino D13 and LED C (Cathode) to Arduino GND.

Connect Button 1.r to Arduino D2 and Button 2.r to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Click and hold the button on the canvas to see the LED light up.

Expected Output

Serial Monitor:

Button ON/OFF Ready Button Pressed - LED ON Button Pressed - LED ON ...

Expected Canvas Behavior

| Button Action | Pin D2 Reading | Pin D13 Output | LED State |
| --- | --- | --- | --- |
| Released | HIGH (5V) | LOW (0V) | OFF |
| Pressed | LOW (0V) | HIGH (5V) | ON |

The LED on the canvas lights up only when the button is held down.

Code Walkthrough

| Line / Block | What It Does |
| --- | --- |
| pinMode(BUTTON_PIN, INPUT_PULLUP) | Configures pin D2 as an input and enables a built-in pull-up resistor. This keeps the pin state at HIGH (5V) until the button is pressed, pulling it to LOW (GND). |
| digitalRead(BUTTON_PIN) | Reads the current voltage level of pin D2, returning either HIGH or LOW. |
| if (buttonState == LOW) | Checks if the button is pressed. Because we are using pull-up logic, a pressed button connects the pin to GND (LOW). |

Hardware & Safety Concept: Pull-up Resistors

If you configure a pin as a simple INPUT without a resistor, the pin is "floating". It behaves like an antenna, picking up stray electromagnetic noise and randomly switching between HIGH and LOW. A pull-up resistor connects the pin to 5V to ensure it remains stable at HIGH when the button is open.

Try This! (Challenges)

Reverse Logic: Modify the code so that the LED is normally ON, and turns OFF only when you press the button.

Add Release Message: Print "Button Released" to the Terminal exactly once when the button is let go.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| LED stays on constantly | Button wired to VCC/5V instead of GND | Connect button pin 2.r to GND. |
| Button doesn't turn on LED | Missing INPUT_PULLUP configuration | Check setup() to make sure you wrote INPUT_PULLUP instead of INPUT. |
| Terminal shows garbled text | Baud rate mismatch | Ensure the Terminal baud rate is set to 9600. |

Mode Notes

These patterns (pinMode, digitalRead, digitalWrite, conditional tests) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[01 - LED Blink](01-led-blink.md)

[12 - Button Toggle](12-button-toggle.md)