12 - Button Toggle

Toggle an LED on and off with a single button press.

Goal

Learn how to use state tracking variables to save the status of an output, allowing a single button click to toggle between ON and OFF states instead of requiring the button to be held down.

What You Will Build

Pressing the button once turns the LED on. The LED stays on after you release the button. Pressing the button a second time turns the LED off.

Why D2 and D13? Pin D2 handles input tracking while pin D13 drives the output state visually on both the canvas and the onboard LED.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| Push Button | button | Yes | Yes |
| LED | led | Yes | Yes |
| 220 ohm resistor | resistor | Optional | Yes |

Wiring

| Component | Component Pin | Arduino Pin | Notes |
| --- | --- | --- | --- |
| Push Button | 1.r | D2 | Input signal connection |
| Push Button | 2.r | GND | Ground reference |
| LED | A (Anode, longer leg) | D13 | Output signal connection |
| LED | C (Cathode, shorter leg) | GND | Ground reference |

Code

const int BUTTON_PIN = 2; const int LED_PIN = 13; int ledState = LOW; // Track the current state of the LED (HIGH or LOW) int lastButtonState = HIGH; // Track the previous state of the button void setup() { pinMode(BUTTON_PIN, INPUT_PULLUP); pinMode(LED_PIN, OUTPUT); digitalWrite(LED_PIN, ledState); Serial.begin(9600); Serial.println("Button Toggle Ready"); } void loop() { int buttonState = digitalRead(BUTTON_PIN); // Check if the button transition changed from HIGH to LOW (just pressed) if (buttonState == LOW && lastButtonState == HIGH) { // Toggle the LED state variable if (ledState == LOW) { ledState = HIGH; Serial.println("Toggle - ON"); } else { ledState = LOW; Serial.println("Toggle - OFF"); } digitalWrite(LED_PIN, ledState); // Simple software debounce delay to filter mechanical noise delay(150); } // Save the current state for the next comparison lastButtonState = buttonState; }

What to Click in MbedO

Drag Arduino Uno, Push Button, and LED onto the canvas.

Connect LED A (Anode) to Arduino D13 and LED C (Cathode) to Arduino GND.

Connect Button 1.r to Arduino D2 and Button 2.r to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Click the button on the canvas once to turn the LED on, and click it again to turn it off.

Expected Output

Serial Monitor:

Button Toggle Ready Toggle - ON Toggle - OFF ...

Expected Canvas Behavior

| Button Event | lastButtonState | buttonState | ledState Result | LED Pin D13 |
| --- | --- | --- | --- | --- |
| Default | HIGH | HIGH | LOW | LOW (0V) - OFF |
| Click Starts | HIGH | LOW | HIGH | HIGH (5V) - ON |
| Click Held | LOW | LOW | HIGH (no change) | HIGH (5V) - ON |
| Click Released | LOW | HIGH | HIGH (no change) | HIGH (5V) - ON |
| Click 2 Starts | HIGH | LOW | LOW | LOW (0V) - OFF |

The LED state changes only on the transition edge of a button press.

Code Walkthrough

| Line / Variable | What It Does |
| --- | --- |
| int ledState = LOW; | Declares a state variable. We write this variable to the LED pin to remember the ON/OFF state between loops. |
| lastButtonState | Remembers what the button was reading in the previous loop execution, allowing edge-detection. |
| if (buttonState == LOW && lastButtonState == HIGH) | This is only true during the exact transition from unpressed (HIGH) to pressed (LOW). It prevents the code inside from looping continuously while the button is held down. |

Hardware & Safety Concept: Contact Bounce

Mechanical buttons contain small metal leaves. When pressed, these contacts strike each other and bounce slightly for a few milliseconds before forming a stable contact. Microcontrollers are so fast that they detect these bounces as multiple rapid presses, causing the LED to toggle on and off randomly. A debounce delay (delay(150)) bypasses this noise by ignoring any inputs immediately following the first transition.

Try This! (Challenges)

Reduce Debounce: Change delay(150) to delay(5). Press the button repeatedly. Does it still toggle cleanly every time?

Beep on Toggle: Wire a Buzzer to D8 and generate a short chirp (tone(8, 2000, 50);) only when the LED toggles ON.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| LED toggles rapidly when the button is held | Missing lastButtonState check | Ensure you check both buttonState == LOW and lastButtonState == HIGH in the conditional test. |
| LED does not change state | Debounce time too high or bad wiring | Verify D2 is connected to the button and the code has delay(150). |

Mode Notes

These patterns (state variables, transition logic, and simple conditional branches) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[11 - Button ON/OFF](11-button-onoff.md)

[13 - Button Counter Print](13-button-counter-print.md)