13 - Button Counter Print

Count how many times a button is pressed and display the count in the Terminal.

Goal

Learn how to increment a counter variable based on button state transitions and print variable values dynamically to the Terminal.

What You Will Build

Each time you press the push button, the counter increments by one, and a message displaying the total count is printed to the Terminal.

Why D2? Pin D2 is configured as our digital input pin with pull-up logic to detect button clicks reliably.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| Push Button | button | Yes | Yes |

Wiring

| Component | Component Pin | Arduino Pin | Notes |
| --- | --- | --- | --- |
| Push Button | 1.r | D2 | Input signal connection |
| Push Button | 2.r | GND | Ground reference |

Code

const int BUTTON_PIN = 2; int buttonPushCounter = 0; // Stores the total number of button presses int lastButtonState = HIGH; // Track the previous state of the button void setup() { pinMode(BUTTON_PIN, INPUT_PULLUP); Serial.begin(9600); Serial.println("Button Counter Ready"); } void loop() { int buttonState = digitalRead(BUTTON_PIN); // Check if the button transition changed from HIGH to LOW (just pressed) if (buttonState == LOW && lastButtonState == HIGH) { buttonPushCounter = buttonPushCounter + 1; Serial.print("Number of button pushes: "); Serial.println(buttonPushCounter); delay(150); // Software debounce delay } lastButtonState = buttonState; }

What to Click in MbedO

Drag Arduino Uno and Push Button onto the canvas.

Connect Button 1.r to Arduino D2 and Button 2.r to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Open the Terminal tab in the bottom dock.

Click the button on the canvas multiple times to see the count increase.

Expected Output

Terminal:

Button Counter Ready Number of button pushes: 1 Number of button pushes: 2 ...

Expected Canvas Behavior

| Action | lastButtonState | buttonState | counter value | Terminal Printout |
| --- | --- | --- | --- | --- |
| Start | HIGH | HIGH | 0 | No new print |
| 1st Click | HIGH | LOW | 1 | "Number of button pushes: 1" |
| Release | LOW | HIGH | 1 | No new print |
| 2nd Click | HIGH | LOW | 2 | "Number of button pushes: 2" |

The count increments exactly once per click.

Code Walkthrough

| Line / Statement | What It Does |
| --- | --- |
| buttonPushCounter = buttonPushCounter + 1; | Increments the counter variable by 1. Can also be written as buttonPushCounter++;. |
| Serial.print("...") | Prints the text message inside quotes without moving to a new line. |
| Serial.println(buttonPushCounter) | Prints the value of the counter and adds a new line character, forcing the next print statement to start below. |

Hardware & Safety Concept: Digital Logic Levels

A digital input pin only resolves two discrete states: HIGH (representing 5V) or LOW (representing 0V). When using INPUT_PULLUP, the pin reads HIGH (logic level 1) when the switch is open. Pressing the button shorts the pin directly to Ground, forcing it to read LOW (logic level 0).

Try This! (Challenges)

Reset Button: Wire a second button to D3 and modify the code so that pressing this second button resets the counter back to 0 and prints "Counter Reset".

Milestone Alerts: Add an if statement to check if the count is a multiple of 5 (using the modulo operator: if (buttonPushCounter % 5 == 0)) and print a special milestone message.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Counter jumps up by multiple numbers on a single press | Debounce delay missing | Make sure delay(150) is inside the conditional block. |
| Counter prints continuously without pressing the button | Button wired incorrectly or missing INPUT_PULLUP | Verify wiring (D2 and GND) and check that INPUT_PULLUP is declared in setup(). |

Mode Notes

These patterns (integer arithmetic, edge detection, and print statements) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[12 - Button Toggle](12-button-toggle.md)

[15 - Analog Meter Serial](15-analog-meter-serial.md)