14 - Dial LED Brightness

Control the brightness of an LED using a potentiometer knob.

Goal

Learn how to read an analog input voltage from a potentiometer (0 to 5V), scale that reading to a PWM duty cycle (0 to 255) using map(), and drive LED brightness accordingly.

What You Will Build

Turning the potentiometer dial on the canvas changes the LED brightness smoothly from fully off to fully bright.

Why A0 and D9? Pin A0 connects to the built-in 10-bit Analog-to-Digital Converter (ADC). Pin D9 is a dedicated PWM output channel required to drive varying LED brightness.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| Potentiometer | potentiometer | Yes | Yes |
| LED | led | Yes | Yes |
| 220 ohm resistor | resistor | Optional | Yes |

Wiring

| Component | Component Pin | Arduino Pin | Notes |
| --- | --- | --- | --- |
| Potentiometer | 1 | 5V | Positive voltage reference |
| Potentiometer | 2 (Wiper) | A0 | Analog signal connection |
| Potentiometer | 3 | GND | Ground reference |
| LED | A (Anode, longer leg) | D9 | Output signal connection (must be PWM pin) |
| LED | C (Cathode, shorter leg) | GND | Ground reference |

Code

const int POT_PIN = A0; const int LED_PIN = 9; // Must be a PWM pin (3, 5, 6, 9, 10, 11) void setup() { pinMode(LED_PIN, OUTPUT); Serial.begin(9600); Serial.println("Dial Brightness Ready"); } void loop() { // Read potentiometer value (returns 0 to 1023) int val = analogRead(POT_PIN); // Scale value to LED duty cycle range (0 to 255) int brightness = map(val, 0, 1023, 0, 255); // Write brightness duty cycle to LED analogWrite(LED_PIN, brightness); Serial.print("Pot: "); Serial.print(val); Serial.print(" - Brightness: "); Serial.println(brightness); delay(100); // 100 ms refresh delay }

What to Click in MbedO

Drag Arduino Uno, Potentiometer, and LED onto the canvas.

Connect Potentiometer pin 1 to Arduino 5V, pin 2 (wiper) to Arduino A0, and pin 3 to Arduino GND.

Connect LED A to Arduino D9 (a PWM pin) and LED C to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Double-click the Potentiometer to open its properties, adjust the slider/knob value, and watch the LED brightness change.

Expected Output

Serial Monitor:

Dial Brightness Ready Pot: 0 - Brightness: 0 Pot: 512 - Brightness: 127 Pot: 1023 - Brightness: 255 ...

Expected Canvas Behavior

| Potentiometer Angle | Wiper Voltage | Pin A0 Reading | mapped Duty Cycle | LED State |
| --- | --- | --- | --- | --- |
| 0% (Counter-clockwise) | 0.0V | 0 | 0 | OFF |
| 50% (Middle) | 2.5V | 512 | 127 | Medium Bright |
| 100% (Clockwise) | 5.0V | 1023 | 255 | Full Bright |

The LED on the canvas alters brightness smoothly as the potentiometer dial is turned.

Code Walkthrough

| Line / Statement | What It Does |
| --- | --- |
| analogRead(POT_PIN) | Reads the voltage on pin A0. The Arduino ADC converts this voltage (0 to 5V) into a whole number between 0 (for 0V) and 1023 (for 5V). |
| map(val, 0, 1023, 0, 255) | Scales the 10-bit input scale (0-1023) down to the 8-bit output scale (0-255) needed for PWM output. |
| analogWrite(LED_PIN, brightness) | Pushes a PWM wave of the mapped duty cycle to D9, altering the power delivered to the LED. |

Hardware & Safety Concept: Analog vs Digital Signals

A digital signal has only two states: ON (5V) or OFF (0V).

An analog signal can vary continuously to any voltage between 0V and 5V.

A potentiometer acts as an adjustable voltage divider. Turning the knob sweeps the center wiper pin across a resistive track, outputting a variable voltage. The Arduino ADC measures this voltage and maps it to a digital representation.

Try This! (Challenges)

Reverse Knob Direction: Modify the map() arguments so that turning the potentiometer clockwise *dims* the LED instead of brightening it. (Hint: change the target range to 255, 0).

Visual Step Meter: Print a series of dashes to the Terminal representing the dial level.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| LED stays fully on or off | LED is on a non-PWM pin | Move the LED anode wire from its current pin to D9, D10, or D11. |
| Potentiometer reads constant 0 or 1023 | Outer pins not connected to 5V/GND | Make sure potentiometer pin 1 connects to 5V and pin 3 connects to GND. |

Mode Notes

These patterns (analogRead mapped directly to analogWrite) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[02 - LED Fade](02-led-fade.md)

[15 - Analog Meter Serial](15-analog-meter-serial.md)