15 - Analog Meter Serial

Display a real-time analog voltage and percentage readout in the Terminal.

Goal

Learn how to read an analog input pin, convert the raw ADC value (0-1023) into a voltage value (0.0-5.0V) and a percentage (0-100%), and print them cleanly to the Terminal.

What You Will Build

As you turn the potentiometer knob on the canvas, the Terminal updates every 200 ms with the raw value, the calculated voltage, and the percentage rotation.

Why A0? Pin A0 feeds directly into the Arduino's built-in analog multiplexer and ADC, which translates analog voltages into digital numbers.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| Potentiometer | potentiometer | Yes | Yes |

Wiring

| Component | Component Pin | Arduino Pin | Notes |
| --- | --- | --- | --- |
| Potentiometer | 1 | 5V | Positive voltage reference |
| Potentiometer | 2 (Wiper) | A0 | Analog signal connection |
| Potentiometer | 3 | GND | Ground reference |

Code

const int POT_PIN = A0; void setup() { Serial.begin(9600); Serial.println("Analog Meter Ready"); } void loop() { // Read raw ADC value (0 to 1023) int rawVal = analogRead(POT_PIN); // Calculate voltage (0.0V to 5.0V) // Float casting ensures decimal math is used float voltage = rawVal * (5.0 / 1023.0); // Calculate percentage (0% to 100%) int percent = map(rawVal, 0, 1023, 0, 100); // Print formatted output to Terminal Serial.print("Raw: "); Serial.print(rawVal); Serial.print(" ¦ Volts: "); Serial.print(voltage); Serial.print("V ¦ Percent: "); Serial.print(percent); Serial.println("%"); delay(200); }

What to Click in MbedO

Drag Arduino Uno and Potentiometer onto the canvas.

Connect Potentiometer pin 1 to Arduino 5V, pin 2 (wiper) to Arduino A0, and pin 3 to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Open the Terminal tab in the bottom dock.

Double-click the Potentiometer to adjust the knob and watch the numbers update in the Terminal.

Expected Output

Terminal:

Analog Meter Ready Raw: 0 ¦ Volts: 0.00V ¦ Percent: 0% Raw: 512 ¦ Volts: 2.50V ¦ Percent: 50% Raw: 1023 ¦ Volts: 5.00V ¦ Percent: 100% ...

Expected Canvas Behavior

| Potentiometer Input | Wiper Voltage | Pin A0 Reading | Calculated Float | Printed Percent |
| --- | --- | --- | --- | --- |
| 0% | 0.0V | 0 | 0.00 | 0% |
| 25% | 1.25V | 256 | 1.25 | 25% |
| 50% | 2.50V | 512 | 2.50 | 50% |
| 100% | 5.00V | 1023 | 5.00 | 100% |

The values update in real-time as the slider moves.

Code Walkthrough

| Line / Expression | What It Does |
| --- | --- |
| float voltage = rawVal * (5.0 / 1023.0) | Converts the raw 10-bit integer (0-1023) into its voltage equivalent. Since the Arduino Uno runs on 5V, dividing 5.0 by 1023.0 gives the voltage step per unit of ADC value. |
| float | A data type that stores decimal numbers (floating-point numbers). This is essential for printing voltages like 2.50V. |
| map(rawVal, 0, 1023, 0, 100) | Scales the raw reading directly to a percentage rotation range (0 to 100). |

Hardware & Safety Concept: ADC Resolution

The Arduino Uno uses a 10-bit Analog-to-Digital Converter (ADC). This means it divides the range between 0V and 5V into 1024 steps. The smallest voltage change the Arduino can detect is:

5V / 1023 = 0.00489V (approx. 4.89 mV).

This is the resolution limit of your analog inputs.

Try This! (Challenges)

Low Voltage Warning: Add a conditional statement (if) that prints a warning message "Low Battery Alert!" when the calculated voltage drops below 1.5V.

Simple Bar Graph: Write a conditional block that prints [===  ] if the percentage is between 40% and 60%, and [=====] if above 80%, creating a simple text-based level bar.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Voltage shows only 0.00V or 5.00V | Wiper pin not wired to A0 | Check that the center pin of the potentiometer is wired to A0 (not the outer pins). |
| Values bounce up and down constantly | Floating input | Make sure the outer pins are wired firmly to 5V and GND. If they are loose, electrical noise will cause wild readings. |

Mode Notes

These patterns (floating-point calculations, basic math, and serial prints) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[13 - Button Counter Print](13-button-counter-print.md)

[14 - Dial LED Brightness](14-dial-led-brightness.md)