16 - Touch ON/OFF

Control an LED using a TTP223 Capacitive Touch Sensor.

Goal

Learn how to read an active capacitive touch sensor as a digital input and use it to control an LED output.

What You Will Build

Touching the active sensor pad on the canvas turns the LED on instantly. Releasing your touch turns the LED off.

Why D2 and D13? Pin D2 reads the active-high digital signal from the sensor. Pin D13 drives the visual indicator LED.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| TTP223 Touch | ttp223_touch | Yes | Yes |
| LED | led | Yes | Yes |
| 220 ohm resistor | resistor | Optional | Yes |

Wiring

| Component | Component Pin | Arduino Pin | Notes |
| --- | --- | --- | --- |
| Touch Sensor | SIG (Signal) | D2 | Input signal connection |
| Touch Sensor | VCC | 5V | Power supply (5V) |
| Touch Sensor | GND | GND | Ground reference |
| LED | A (Anode, longer leg) | D13 | Output signal connection |
| LED | C (Cathode, shorter leg) | GND | Ground reference |

Code

const int TOUCH_PIN = 2; const int LED_PIN = 13; void setup() { // Configured as INPUT because TTP223 module outputs a driven 5V (HIGH) signal pinMode(TOUCH_PIN, INPUT); pinMode(LED_PIN, OUTPUT); Serial.begin(9600); Serial.println("Touch Sensor Ready"); } void loop() { int touchState = digitalRead(TOUCH_PIN); if (touchState == HIGH) { digitalWrite(LED_PIN, HIGH); Serial.println("Touch Detected - LED ON"); } else { digitalWrite(LED_PIN, LOW); } delay(100); }

What to Click in MbedO

Drag Arduino Uno, TTP223 Touch Sensor, and LED onto the canvas.

Connect Touch Sensor SIG to Arduino D2, VCC to Arduino 5V, and GND to Arduino GND.

Connect LED A (Anode) to Arduino D13 and LED C (Cathode) to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Click the Touch Sensor on the canvas to activate it (simulating touch) and watch the LED respond.

Expected Output

Terminal:

Touch Sensor Ready Touch Detected - LED ON Touch Detected - LED ON ...

Expected Canvas Behavior

| Action | Touch Sensor state | SIG Pin Reading (D2) | LED State (D13) |
| --- | --- | --- | --- |
| Unpressed | Idle | LOW (0V) | OFF |
| Pressed / Held | Active | HIGH (5V) | ON |

The LED lights up immediately upon contact with the touch sensor.

Code Walkthrough

| Line / Statement | What It Does |
| --- | --- |
| pinMode(TOUCH_PIN, INPUT) | Configures pin D2 as a standard digital input. We do not use pull-up mode because the sensor chip active-drives the pin to HIGH or LOW. |
| digitalRead(TOUCH_PIN) | Reads the active voltage state. Returns HIGH when the plate is touched, and LOW when idle. |

Hardware & Safety Concept: Capacitive Sensing

Mechanical switches require physical force to bridge metal contacts. A capacitive touch sensor works by measuring changes in electrical capacitance. Your body has a natural capacitance; when your finger gets close to the copper pad on the sensor board, it alters the electrical charge of the pad. The onboard TTP223 chip detects this shift and outputs a digital HIGH signal.

Try This! (Challenges)

Touch Toggle: Modify the code using state variables (like [12 - Button Toggle](12-button-toggle.md)) so that touching the sensor once turns the LED on, and it stays on until touched again.

Double Alert tone: Play a quick sound beep when touch is first detected.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| LED stays on constantly | Incorrect pin mode configuration | Ensure TOUCH_PIN is configured as INPUT (not INPUT_PULLUP). Check that SIG is wired to D2. |
| Sensor doesn't respond | VCC or GND wire missing | The active touch sensor module requires power to operate. Make sure VCC is wired to 5V and GND is connected to ground. |

Mode Notes

These patterns (digitalRead and simple conditional checks) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[11 - Button ON/OFF](11-button-onoff.md)

[12 - Button Toggle](12-button-toggle.md)