18 - Light Meter

Measure ambient light levels using a Light Dependent Resistor (LDR) and display them.

Goal

Learn how to read an analog light sensor, understand how photoresistors behave under light, and print raw sensor values to the Terminal.

What You Will Build

As you change the light level slider on the LDR sensor, the Terminal prints the updated raw reading (0 to 1023) every 200 ms.

Why A0? The LDR sensor output is an analog voltage that must be read by the Arduino's ADC. Pin A0 is configured to perform this conversion.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| LDR Light Sensor | ldr | Yes | Yes |
| 10k ohm resistor | resistor | Optional | Yes (required for discrete wiring) |

Wiring

| Component | Component Pin | Arduino Pin | Notes |
| --- | --- | --- | --- |
| LDR Sensor | VCC | 5V | Power supply (5V) |
| LDR Sensor | AO (Analog Out) | A0 | Analog signal connection |
| LDR Sensor | GND | GND | Ground reference |

Code

const int LDR_PIN = A0; void setup() { Serial.begin(9600); Serial.println("LDR Light Meter Ready"); } void loop() { // Read raw light sensor value (0 to 1023) int lightLevel = analogRead(LDR_PIN); Serial.print("Light Level (Raw): "); Serial.println(lightLevel); delay(200); // Read 5 times per second }

What to Click in MbedO

Drag Arduino Uno and LDR Light Sensor onto the canvas.

Connect LDR VCC to Arduino 5V, AO to Arduino A0, and GND to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Open the Terminal tab in the bottom dock.

Double-click the LDR sensor on the canvas, adjust the ambient light slider, and watch the values change in the Terminal.

Expected Output

Terminal:

LDR Light Meter Ready Light Level (Raw): 120 Light Level (Raw): 540 Light Level (Raw): 980 ...

Expected Canvas Behavior

| Ambient Light Slider | Sensor Resistance | Signal Voltage | Pin A0 Reading |
| --- | --- | --- | --- |
| Dark | Very High | Near 0V | Low (e.g. < 200) |
| Indoor Light | Moderate | Near 2.5V | Medium (e.g. 500) |
| Direct Sunlight | Very Low | Near 5.0V | High (e.g. > 900) |

The raw output matches the slider value directly in real-time.

Code Walkthrough

| Line / Statement | What It Does |
| --- | --- |
| analogRead(LDR_PIN) | Measures the analog voltage output of the sensor module and returns an integer from 0 (darkest) to 1023 (brightest). |

Hardware & Safety Concept: Photoresistors (LDRs)

A Light Dependent Resistor (LDR) changes its electrical resistance based on the amount of light falling on its surface.

In darkness, the resistance is extremely high (often exceeding 1 Megaohm).

In bright light, the resistance drops significantly (down to a few hundred ohms).

By combining the LDR with a fixed resistor in a voltage divider circuit on the sensor breakout board, this changing resistance is converted to a variable voltage that the Arduino ADC can measure.

Try This! (Challenges)

Light Percentage: Scale the output using map(lightLevel, 0, 1023, 0, 100) to display a percentage scale from 0% (dark) to 100% (bright).

Low Light Alert: Add an if statement to print a warning message "Night Mode Active" to the Terminal when the light level falls below 300.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Readings are stuck at 0 or 1023 | Wiring issue | Make sure LDR VCC is connected to 5V, GND to GND, and AO to A0. |
| Values do not change when slider is adjusted | Wrong pin configured in code | Ensure the LDR_PIN constant matches the A0 pin you wired the sensor to. |

Mode Notes

These patterns (analog input and serial logging) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[15 - Analog Meter Serial](15-analog-meter-serial.md)

[19 - Night Detector](19-night-detector.md)