19 - Night Detector

Create an automated night light that turns an LED on when it gets dark.

Goal

Learn how to use an LDR's analog reading to trigger a digital output (LED) dynamically based on a custom threshold limit.

What You Will Build

When the ambient light level drops below a set threshold (simulating nightfall), the LED turns on automatically. When light levels rise, the LED turns off.

Why A0 and D13? Pin A0 tracks the analog light levels continuously. Pin D13 drives the visual indicator LED (simulating a street lamp).

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| LDR Light Sensor | ldr | Yes | Yes |
| LED | led | Yes | Yes |
| 220 ohm resistor | resistor | Optional | Yes |

Wiring

| Component | Component Pin | Arduino Pin | Notes |
| --- | --- | --- | --- |
| LDR Sensor | VCC | 5V | Power supply (5V) |
| LDR Sensor | AO | A0 | Analog signal connection |
| LDR Sensor | GND | GND | Ground reference |
| LED | A (Anode, longer leg) | D13 | Output signal connection |
| LED | C (Cathode, shorter leg) | GND | Ground reference |

Code

const int LDR_PIN = A0; const int LED_PIN = 13; // Threshold below which we consider it "dark" (0 to 1023) const int DARK_THRESHOLD = 400; void setup() { pinMode(LED_PIN, OUTPUT); Serial.begin(9600); Serial.println("Night Detector Ready"); } void loop() { int lightLevel = analogRead(LDR_PIN); Serial.print("Light Level: "); Serial.println(lightLevel); // If the light level is below our threshold, turn ON the LED if (lightLevel < DARK_THRESHOLD) { digitalWrite(LED_PIN, HIGH); Serial.println("Night detected - LED ON"); } else { digitalWrite(LED_PIN, LOW); } delay(500); // Check twice per second }

What to Click in MbedO

Drag Arduino Uno, LDR Light Sensor, and LED onto the canvas.

Connect LDR VCC to Arduino 5V, AO to Arduino A0, and GND to Arduino GND.

Connect LED A (Anode) to Arduino D13 and LED C (Cathode) to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Double-click the LDR Sensor on the canvas, move the slider to low light (below 400), and watch the LED turn on.

Expected Output

Terminal:

Night Detector Ready Light Level: 800 Light Level: 350 Night detected - LED ON ...

Expected Canvas Behavior

| Ambient Light State | Pin A0 Reading | Condition check | LED State (D13) |
| --- | --- | --- | --- |
| Bright (Day) | > 400 | False | LOW - OFF |
| Dark (Night) | < 400 | True | HIGH - ON |

The LED activates immediately when the slider falls below the threshold.

Code Walkthrough

| Line / Expression | What It Does |
| --- | --- |
| const int DARK_THRESHOLD = 400; | Defines our switching point. Lower values represent darkness, and higher values represent daylight. |
| if (lightLevel < DARK_THRESHOLD) | Evaluates if the current light level is below the threshold. If true, the LED is driven HIGH. |

Hardware & Safety Concept: Threshold Hysteresis

In real-world automated systems, if the light level hovers exactly at the threshold value (e.g. 400), the LED might flicker on and off rapidly due to minor voltage noise. Real-world systems use hysteresis (e.g., turn ON at 380, but do not turn OFF until it reaches 420) to prevent this rapid switching and protect the hardware.

Try This! (Challenges)

Calibrate Threshold: Calibrate the system by finding the exact slider value where you want the light to activate, and update DARK_THRESHOLD in the code.

Reverse logic (Solar Indicator): Change the logic so the LED turns ON when it is *bright* (above 800) and turns OFF when it is dark, simulating a solar battery charging indicator.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| LED stays on even in bright light | Threshold is set too high | Lower the DARK_THRESHOLD constant (e.g., from 400 to 200). |
| LED never turns on | Threshold is set too low or wiring error | Raise the DARK_THRESHOLD (e.g. to 600) or check the LED wiring. |

Mode Notes

Conditional testing of an analogRead result against a constant value is supported by MbedO interpreted mode and is safe for this beginner project.

Related Projects

[18 - Light Meter](18-light-meter.md)

[30 - Motion Light](../intermediate/30-motion-light.md)