20 - Thermistor Raw Print

Read raw analog values from an NTC Thermistor and print them to the Terminal.

Goal

Learn how to read an NTC Thermistor as an analog input and understand how physical temperature shifts alter the resistance and output voltage.

What You Will Build

As you change the temperature slider on the thermistor component, the Terminal prints the updated raw ADC value every 200 ms.

Why A0? The thermistor's voltage divider output must connect to an analog input pin (A0) to be parsed by the ADC.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| NTC Thermistor | ntc_temp | Yes | Yes |
| 10k ohm resistor | resistor | Optional | Yes (required for discrete divider) |

Wiring

| Component | Component Pin | Arduino Pin | Notes |
| --- | --- | --- | --- |
| Thermistor | VCC | 5V | Power supply (5V) |
| Thermistor | OUT | A0 | Analog signal connection |
| Thermistor | GND | GND | Ground reference |

Code

const int NTC_PIN = A0; void setup() { Serial.begin(9600); Serial.println("NTC Thermistor Monitor Ready"); } void loop() { // Read raw thermistor value (0 to 1023) int rawVal = analogRead(NTC_PIN); Serial.print("Thermistor Raw Value: "); Serial.println(rawVal); delay(200); // Read 5 times per second }

What to Click in MbedO

Drag Arduino Uno and NTC Thermistor onto the canvas.

Connect Thermistor VCC to Arduino 5V, OUT to Arduino A0, and GND to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Open the Terminal tab in the bottom dock.

Double-click the Thermistor on the canvas, adjust the temperature slider, and watch the values change in the Terminal.

Expected Output

Terminal:

NTC Thermistor Monitor Ready Thermistor Raw Value: 512 Thermistor Raw Value: 420 Thermistor Raw Value: 680 ...

Expected Canvas Behavior

| Temperature State | Sensor Resistance | Signal Voltage | Pin A0 Reading |
| --- | --- | --- | --- |
| Cold | High | High (Near 5V) | High (e.g. > 700) |
| Room Temp | Moderate (10k) | Medium (Near 2.5V) | Medium (e.g. 512) |
| Hot | Low | Low (Near 0V) | Low (e.g. < 300) |

The raw ADC value drops as the temperature rises.

Code Walkthrough

| Line / Statement | What It Does |
| --- | --- |
| analogRead(NTC_PIN) | Reads the analog output voltage from the thermistor's onboard voltage divider. |

Hardware & Safety Concept: Negative Temperature Coefficient (NTC)

"NTC" stands for Negative Temperature Coefficient. This means that as temperature *increases*, the thermistor's electrical resistance *decreases*.

The module's built-in voltage divider circuit outputs a voltage that is inversely proportional to the temperature.

Therefore, a higher temperature yields a lower analog voltage, resulting in a lower raw ADC value.

Try This! (Challenges)

Temperature Trend Indicator: Add logic to compare the current reading to the previous reading and print whether the temperature is "Getting Hotter" or "Getting Colder".

Freeze Alert: Add an if statement to print a warning message "Freeze Warning!" when the raw value rises above 900 (which corresponds to cold temperatures).

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Value stays stuck at 0 or 1023 | Wiring error | Ensure OUT is connected to A0, VCC to 5V, and GND to GND. |
| Values increase when it gets colder | Normal NTC behavior | This is correct for NTC thermistors. To invert the scale in code, you can subtract the reading from 1023 (1023 - rawVal). |

Mode Notes

These patterns (analog input reading and basic serial prints) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[15 - Analog Meter Serial](15-analog-meter-serial.md)

[41 - Thermostat Relay](../intermediate/41-thermostat-relay.md)