21 - Motion Alert Print

Detect movement using a PIR Motion Sensor and print alerts to the Terminal.

Goal

Learn how to read a PIR (Passive Infrared) motion sensor as a digital input and print a status message only when motion starts or stops.

What You Will Build

When the PIR sensor detects motion, an alert message is printed to the Terminal. When the motion ceases, a quiet message is printed.

Why D2? Pin D2 supports digital input and hardware state checking, making it a standard choice for motion events.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| PIR Motion Sensor | pir | Yes | Yes |

Wiring

| Component | Component Pin | Arduino Pin | Notes |
| --- | --- | --- | --- |
| PIR Sensor | VCC | 5V | Power supply (5V) |
| PIR Sensor | OUT | D2 | Digital signal connection |
| PIR Sensor | GND | GND | Ground reference |

Code

const int PIR_PIN = 2; int lastMotionState = LOW; // Track the previous state of the PIR sensor void setup() { pinMode(PIR_PIN, INPUT); // PIR outputs digital HIGH/LOW Serial.begin(9600); Serial.println("PIR Motion Monitor Ready"); } void loop() { int motionState = digitalRead(PIR_PIN); // If the motion state has changed from the previous loop run if (motionState != lastMotionState) { if (motionState == HIGH) { Serial.println("[ALERT] Motion Detected!"); } else { Serial.println("[INFO] Motion Ended."); } // Update the state history lastMotionState = motionState; } delay(100); }

What to Click in MbedO

Drag Arduino Uno and PIR Motion Sensor onto the canvas.

Connect PIR VCC to Arduino 5V, OUT to Arduino D2, and GND to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Open the Terminal tab in the bottom dock.

Click the PIR sensor on the canvas to toggle its "Motion" property ON and OFF, and watch the alerts appear.

Expected Output

Terminal:

PIR Motion Monitor Ready [ALERT] Motion Detected! [INFO] Motion Ended. ...

Expected Canvas Behavior

| Canvas Motion Property | Pin D2 Reading | comparison check | Terminal Output |
| --- | --- | --- | --- |
| False (Idle) | LOW (0V) | state changed | "[INFO] Motion Ended." |
| True (Moving) | HIGH (5V) | state changed | "[ALERT] Motion Detected!" |

Output is triggered exactly once per motion state shift.

Code Walkthrough

| Line / Statement | What It Does |
| --- | --- |
| int motionState = digitalRead(PIR_PIN) | Reads the digital output of the PIR sensor (HIGH = motion, LOW = quiet). |
| if (motionState != lastMotionState) | Checks for a state transition to prevent printing continuous messages while the state remains the same. |

Hardware & Safety Concept: Passive Infrared (PIR)

A PIR sensor works by measuring infrared (heat) radiation from objects in its field of view. All warm bodies emit infrared radiation. When a person or animal moves in front of the sensor, it detects the change in infrared heat levels across its dual sensor slots and drives the output pin HIGH.

Try This! (Challenges)

LED Alarm: Wire an LED to D13 and modify the code so that the LED turns ON when motion is detected and OFF when it ends.

Move Counter: Add an integer variable to count the total number of movements detected, and print it alongside each alert.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Terminal floods with alerts repeatedly | Missing state change check | Make sure you compare motionState != lastMotionState and save the state with lastMotionState = motionState. |
| Sensor doesn't trigger | VCC or GND wire missing | The PIR sensor requires 5V power to operate. Verify VCC and GND connections. |

Mode Notes

These patterns (digital reads and state change checking) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[11 - Button ON/OFF](11-button-onoff.md)

[30 - Motion Light](../intermediate/30-motion-light.md)