23 - Knock Detector

Detect physical taps or knocks using a Piezo Vibration Sensor and light up an LED.

Goal

Learn how to read a piezo sensor as an analog input to detect mechanical vibrations and trigger a digital output when a set threshold limit is exceeded.

What You Will Build

Tapping the piezo sensor on the canvas generates a voltage spike. If the tap is strong enough to exceed the threshold limit, the LED on pin D13 flashes ON for 100 ms.

Why A0 and D13? Pin A0 measures the fast transient voltage spikes from the piezo element. Pin D13 drives the visual notification LED.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| Piezo Sensor | piezo_sensor | Yes | Yes |
| LED | led | Yes | Yes |
| 1M ohm resistor | resistor | Optional | Yes (crucial protection resistor) |

Wiring

| Component | Component Pin | Arduino Pin | Notes |
| --- | --- | --- | --- |
| Piezo Sensor | POS (Positive) | A0 | Analog signal connection |
| Piezo Sensor | NEG (Negative) | GND | Ground reference |
| LED | A (Anode, longer leg) | D13 | Output signal connection |
| LED | C (Cathode, shorter leg) | GND | Ground reference |

> On physical hardware, you should place a 1 Megaohm resistor in parallel with the piezo sensor (connecting POS directly to NEG) to absorb excess voltage spikes.

Code

const int PIEZO_PIN = A0; const int LED_PIN = 13; // Threshold for what we consider a "knock" (0 to 1023) const int KNOCK_THRESHOLD = 100; void setup() { pinMode(LED_PIN, OUTPUT); Serial.begin(9600); Serial.println("Knock Detector Ready"); } void loop() { int sensorVal = analogRead(PIEZO_PIN); // Check if vibration strength exceeds our threshold limit if (sensorVal > KNOCK_THRESHOLD) { digitalWrite(LED_PIN, HIGH); Serial.print("Knock Detected! Strength: "); Serial.println(sensorVal); delay(100); // Keep LED on briefly so it is visible digitalWrite(LED_PIN, LOW); } }

What to Click in MbedO

Drag Arduino Uno, Piezo Sensor, and LED onto the canvas.

Connect Piezo POS to Arduino A0 and Piezo NEG to Arduino GND.

Connect LED A (Anode) to Arduino D13 and LED C (Cathode) to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Double-click the Piezo Sensor to open its properties, adjust the vibration slider value to exceed 100, and watch the LED flash.

Expected Output

Terminal:

Knock Detector Ready Knock Detected! Strength: 145 Knock Detected! Strength: 420 ...

Expected Canvas Behavior

| Tap Action | Voltage Peak | Pin A0 Reading | Condition check | LED D13 State |
| --- | --- | --- | --- | --- |
| No vibration | 0V | 0 | False | LOW - OFF |
| Gentle Tap | < 0.5V | < 100 | False | LOW - OFF |
| Strong Tap | > 0.5V | > 100 | True | HIGH - ON (100 ms) |

The LED on the canvas flashes instantly when a tap slider value crosses 100.

Code Walkthrough

| Line / Statement | What It Does |
| --- | --- |
| int sensorVal = analogRead(PIEZO_PIN) | Reads the transient voltage generated by the piezo element. |
| if (sensorVal > KNOCK_THRESHOLD) | Evaluates if the vibration strength exceeds the threshold limit. If true, it blinks the LED and prints the tap strength. |

Hardware & Safety Concept: Piezoelectricity

A piezoelectric sensor works by converting mechanical stress (like a tap, bend, or vibration) into electrical energy. When you tap the ceramic crystal inside the piezo element, it deforms slightly and generates a small electrical charge. Because these spikes can exceed 5V and potentially damage the microcontroller input pins, a high-value parallel resistor (e.g. 1 Megaohm) is used on real hardware to bleed off excess charge.

Try This! (Challenges)

Calibrate Sensitivity: Test different tap strengths. Adjust KNOCK_THRESHOLD in the code to make the sensor more or less sensitive.

Knock Alarm: Connect a Buzzer to D8 and play a brief high-pitched tone (tone(8, 2000, 100);) whenever a knock is detected.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| LED flashes constantly | Threshold is set too low | Raise KNOCK_THRESHOLD to a higher number (e.g. 200 or 300) to filter out background vibration noise. |
| LED never flashes | Piezo is connected backwards or wrong threshold | Make sure POS is connected to A0 and NEG is connected to GND. Check that your tap slider value is high enough. |

Mode Notes

These patterns (analog polling and output pulse logic) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[15 - Analog Meter Serial](15-analog-meter-serial.md)

[43 - Knock Doorbell](../intermediate/43-knock-doorbell.md)