24 - Rain Alert

Detect rainfall using a Rain Sensor's digital output and turn on a warning LED.

Goal

Learn how to read a rain sensor's digital threshold output and trigger an LED indicator when water is detected.

What You Will Build

When water drops on the rain sensor board, the digital output drops to LOW (active low), turning on the alert LED on D13. When dry, the LED turns off.

Why D2 and D13? Pin D2 reads the digital threshold alarm output from the comparator chip. Pin D13 drives the alert LED.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| Rain Sensor | rain_sensor | Yes | Yes |
| LED | led | Yes | Yes |
| 220 ohm resistor | resistor | Optional | Yes |

Wiring

| Component | Component Pin | Arduino Pin | Notes |
| --- | --- | --- | --- |
| Rain Sensor | VCC | 5V | Power supply (5V) |
| Rain Sensor | DO (Digital Out) | D2 | Digital signal connection |
| Rain Sensor | GND | GND | Ground reference |
| LED | A (Anode, longer leg) | D13 | Output signal connection |
| LED | C (Cathode, shorter leg) | GND | Ground reference |

Code

const int RAIN_DO_PIN = 2; const int LED_PIN = 13; void setup() { pinMode(RAIN_DO_PIN, INPUT); pinMode(LED_PIN, OUTPUT); Serial.begin(9600); Serial.println("Rain Alarm System Ready"); } void loop() { int rainDetected = digitalRead(RAIN_DO_PIN); // Most rain sensor comparator boards output LOW when wet if (rainDetected == LOW) { digitalWrite(LED_PIN, HIGH); Serial.println("[WARNING] Rain Detected! LED Active"); } else { digitalWrite(LED_PIN, LOW); } delay(300); // Poll 3 times per second }

What to Click in MbedO

Drag Arduino Uno, Rain Sensor, and LED onto the canvas.

Connect Rain Sensor VCC to Arduino 5V, DO to Arduino D2, and GND to Arduino GND.

Connect LED A (Anode) to Arduino D13 and LED C (Cathode) to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Double-click the Rain Sensor on the canvas to open its properties, adjust the "wetness" slider so it crosses the threshold, and watch the LED turn on.

Expected Output

Terminal:

Rain Alarm System Ready [WARNING] Rain Detected! LED Active [WARNING] Rain Detected! LED Active ...

Expected Canvas Behavior

| Moisture Condition | Sensor Grid state | Pin D2 Reading | LED State (D13) |
| --- | --- | --- | --- |
| Dry | Open | HIGH (5V) | LOW - OFF |
| Wet (Rain) | Conductive | LOW (0V) | HIGH - ON |

The LED on the canvas activates immediately when the wetness slider crosses the threshold.

Code Walkthrough

| Line / Statement | What It Does |
| --- | --- |
| int rainDetected = digitalRead(RAIN_DO_PIN) | Reads the digital state of pin D2, which corresponds to the threshold status output from the comparator. |
| if (rainDetected == LOW) | Active-low logic check. When dry, the sensor board outputs 5V (HIGH). When wet, the onboard comparator pulls the line to Ground (LOW). |

Hardware & Safety Concept: Resistive Rain Sensors

A resistive rain sensor consists of a grid of nickel traces on a PCB. When water falls on the board, it acts as a conductor between positive and negative traces, lowering the overall resistance. The module's comparator chip (typically LM393) compares this resistance to a threshold set by an onboard potentiometer. If the wetness exceeds the threshold, the digital output pin DO drops to LOW.

Try This! (Challenges)

Audible Rain Alarm: Wire a Buzzer to D8 and generate a double-chirp alarm tone (tone(8, 1000, 100); delay(150); tone(8, 1000, 100);) only when rain is first detected.

Reverse Output: Adjust the code to work with a sensor configuration that outputs HIGH when wet.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| LED stays on even when dry | Sensitivity threshold is too high | On physical modules, rotate the onboard potentiometer. On the canvas, ensure the slider wetness value is set to 0. |
| Nothing happens when wetness slider is moved | Wrong signal pin wired | Ensure you wired DO (Digital Out) to D2, not AO (Analog Out). |

Mode Notes

These patterns (digital input polling and active-low checking) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[11 - Button ON/OFF](11-button-onoff.md)

[25 - Rain Level Print](25-rain-level-print.md)

[40 - Water Pump Control](../intermediate/40-water-pump-control.md)