25 - Rain Level Print

Measure and display exact rain wetness levels using a Rain Sensor's analog output.

Goal

Learn how to read an analog input from a rain sensor grid, map the raw values to categorize rain intensity, and log the results to the Terminal.

What You Will Build

As you change the wetness slider of the rain sensor, the Terminal displays the raw analog value and a weather description (Dry, Moderate, or Heavy) every 500 ms.

Why A0? The rain sensor grid acts as a variable resistor. The analog voltage output from the divider circuit must connect to A0 to measure exact wetness levels rather than just a simple ON/OFF limit.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| Rain Sensor | rain_sensor | Yes | Yes |

Wiring

| Component | Component Pin | Arduino Pin | Notes |
| --- | --- | --- | --- |
| Rain Sensor | VCC | 5V | Power supply (5V) |
| Rain Sensor | AO (Analog Out) | A0 | Analog signal connection |
| Rain Sensor | GND | GND | Ground reference |

Code

const int RAIN_AO_PIN = A0; void setup() { Serial.begin(9600); Serial.println("Rain Level Monitor Ready"); } void loop() { int rainLevel = analogRead(RAIN_AO_PIN); Serial.print("Rain Wetness (Raw ADC): "); Serial.print(rainLevel); // Note: On physical sensors, a lower value means more water // because water increases conductivity (reduces resistance) if (rainLevel < 300) { Serial.println(" - Heavy Rain!"); } else if (rainLevel < 700) { Serial.println(" - Moderate Rain"); } else { Serial.println(" - Dry / Light Drizzle"); } delay(500); }

What to Click in MbedO

Drag Arduino Uno and Rain Sensor onto the canvas.

Connect Rain Sensor VCC to Arduino 5V, AO to Arduino A0, and GND to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Open the Terminal tab in the bottom dock.

Double-click the Rain Sensor, adjust the "wetness" slider, and watch the values and categories print in the Terminal.

Expected Output

Terminal:

Rain Level Monitor Ready Rain Wetness (Raw ADC): 1023 - Dry / Light Drizzle Rain Wetness (Raw ADC): 520 - Moderate Rain Rain Wetness (Raw ADC): 150 - Heavy Rain! ...

Expected Canvas Behavior

| Rain Sensor State | Output Voltage | Pin A0 Reading | Printed Category |
| --- | --- | --- | --- |
| Dry | Near 5V | > 700 | "Dry / Light Drizzle" |
| Drizzle / Damp | Intermediate | 300 - 700 | "Moderate Rain" |
| Heavy Wetness | Near 0V | < 300 | "Heavy Rain!" |

The printed description changes dynamically as the slider moves.

Code Walkthrough

| Line / Statement | What It Does |
| --- | --- |
| int rainLevel = analogRead(RAIN_AO_PIN) | Reads the analog output voltage from the sensor plate. |
| if (rainLevel < 300) | Active-low check. A wet plate conducts more electricity, bringing the voltage down toward 0V (0). A dry plate has high resistance, keeping the voltage near 5V (1023). |

Hardware & Safety Concept: Sensor Degradation (Electrolysis)

In real-world applications, running a continuous DC current through a resistive sensor exposed to rain and moisture causes rapid metal corrosion due to electrolysis. To prevent this, professional outdoor stations only power the sensor pin right before taking a reading, and immediately turn it off afterward.

Try This! (Challenges)

Calibrated Percentage: Convert the reading so that it shows 0% when dry and 100% when fully wet. (Hint: Use map(rainLevel, 1023, 0, 0, 100)).

Alert beep: Add a Buzzer to D8 and sound a short chirp only when the weather changes from Dry to Moderate Rain.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Reading stays stuck at 1023 even when slider is high | AO pin wired incorrectly | Verify that the AO pin is wired to A0. If you wire DO instead, it will only register 0 or 1023. |
| Code doesn't compile | Wrong pin naming | Make sure A0 is written as A0 (capital letter A and number zero). |

Mode Notes

These patterns (analog reads and multi-branch comparison blocks) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[15 - Analog Meter Serial](15-analog-meter-serial.md)

[24 - Rain Alert](24-rain-alert.md)