27 - Touch Lamp

Build a touch-sensitive lamp that toggles ON and OFF with each touch.

Goal

Learn how to apply toggle state logic to a capacitive touch sensor input, turning it into a latching ON/OFF switch.

What You Will Build

Touching the touch sensor once turns the LED on. The LED stays on when you remove your hand. Touching the sensor a second time turns the LED off.

Why D2 and D13? Pin D2 reads the transition edge of the touch sensor. Pin D13 drives the visual indicator LED.

Parts Needed

| Part | MbedO Component Type | Required in MbedO | Required on hardware |
| --- | --- | --- | --- |
| Arduino Uno | arduino | Yes | Yes |
| TTP223 Touch | ttp223_touch | Yes | Yes |
| LED | led | Yes | Yes |
| 220 ohm resistor | resistor | Optional | Yes |

Wiring

| Component | Component Pin | Arduino Pin | Notes |
| --- | --- | --- | --- |
| Touch Sensor | SIG | D2 | Input signal connection |
| Touch Sensor | VCC | 5V | Power supply (5V) |
| Touch Sensor | GND | GND | Ground reference |
| LED | A (Anode, longer leg) | D13 | Output signal connection |
| LED | C (Cathode, shorter leg) | GND | Ground reference |

Code

const int TOUCH_PIN = 2; const int LED_PIN = 13; int lampState = LOW; // Track if the lamp is ON or OFF int lastTouchState = LOW; // Track the previous reading of the touch sensor void setup() { pinMode(TOUCH_PIN, INPUT); pinMode(LED_PIN, OUTPUT); digitalWrite(LED_PIN, lampState); Serial.begin(9600); Serial.println("Touch Lamp Ready"); } void loop() { int touchState = digitalRead(TOUCH_PIN); // Detect state transition from LOW (no touch) to HIGH (touched) if (touchState == HIGH && lastTouchState == LOW) { if (lampState == LOW) { lampState = HIGH; Serial.println("Lamp - ON"); } else { lampState = LOW; Serial.println("Lamp - OFF"); } digitalWrite(LED_PIN, lampState); delay(200); // Debounce delay } lastTouchState = touchState; }

What to Click in MbedO

Drag Arduino Uno, TTP223 Touch Sensor, and LED onto the canvas.

Connect Touch Sensor SIG to Arduino D2, VCC to Arduino 5V, and GND to Arduino GND.

Connect LED A (Anode) to Arduino D13 and LED C (Cathode) to Arduino GND.

Paste the code into the editor.

Use the default Arduino interpreted runtime.

Click Run.

Click the Touch Sensor once to turn the LED on, and click it again to turn it off.

Expected Output

Terminal:

Touch Lamp Ready Lamp - ON Lamp - OFF ...

Expected Canvas Behavior

| Touch Action | lastTouchState | touchState | lampState Result | LED State (D13) |
| --- | --- | --- | --- | --- |
| Default | LOW | LOW | LOW | LOW - OFF |
| 1st Touch | LOW | HIGH | HIGH | HIGH - ON |
| Hand Removed | HIGH | LOW | HIGH (no change) | HIGH - ON |
| 2nd Touch | LOW | HIGH | LOW | LOW - OFF |

The lamp toggles state exactly on the rising transition edge of the touch event.

Code Walkthrough

| Line / Expression | What It Does |
| --- | --- |
| if (touchState == HIGH && lastTouchState == LOW) | Checks for a rising edge (the exact millisecond contact begins). This prevents the lamp from toggling repeatedly if you hold your finger on the sensor. |

Hardware & Safety Concept: Capacitive Toggle Switch

In consumer electronics, touch lamps use a conductive metal plate connected to a specialized sensing chip. The TTP223 chip handles all the high-frequency measurements to detect changes in electrical capacitance and converts this into a simple, clean digital HIGH signal when touched, making code implementation direct.

Try This! (Challenges)

Brightness presets: Use D9 for the LED and modify the code to cycle through Dim (50), Medium (150), Full (255), and OFF on consecutive touches.

Double chirp: Add a Buzzer to D8 and code a double chirp sound whenever the lamp turns ON.

Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Lamp toggles rapidly when touched | Missing state transition check | Ensure you have lastTouchState = touchState at the end of the loop and you are checking lastTouchState == LOW. |
| LED doesn't light up | Wiring issue | Make sure Touch Sensor SIG is connected to D2, VCC to 5V, and GND to ground. |

Mode Notes

These patterns (state transitions and digital toggles) are supported by MbedO interpreted mode and are safe for this beginner project.

Related Projects

[12 - Button Toggle](12-button-toggle.md)

[16 - Touch ON/OFF](16-touch-onoff.md)